| To: | As per email distribution list. |  |  |  |
| --- | --- | --- | --- | --- |
| cc: |  |  |  |  |
|  |  |  | Pages: (incl. cover sheet): |  |
| From: |  |  | Date: 19. July 2013 |  |

### International Field Test Plan for SikaGrout®-334 (Austria)

Dear Colleagues

We kindly ask you to assist us field test the new cementitious grout SikaGrout®-334.

SikaGrout®-334 is a 1-component, ready to mix, free flowing, expanding, low shrinkage pouring mortar containing recycled waste materials.

The main characteristic of SikaGrout®-334 is the significant reduction of Portland cement which is estimated to generate approximately one tonne of carbon dioxide per produced tonne.  In direct comparison to an equivalent performing SikaGrout® product, reducing the amount of Portland cement in SikaGrout®-334 by 28 % has demonstrated the following benefits:

- ~12 % reduction in primary energy demand (CED)

- ~17 % reduction in carbon dioxide emissions  (GWP)

- ~16 % reduction in  summer smog (POCP)

SikaGrout®-334 is used as free flowing pouring mortar for layer thickness of between 15 mm to 125 mm and is suitable for precision grouting of:

- Heavy equipment / machine bases

- Base plates

- Dowling reinforcing bars

- Bedding joints in pre-cast concrete sections

- Filling cavities, voids, gaps and recesses

- Sealing around penetrations

- Post fixings

The SikaGrout®-334 characteristics are:

- 1 day compressive strength ~35 MPa

- 28 day compressive strength ~80 MPa

- Adhesive bond to concrete > 2.0 MPa after 28 days

- Pot life ~40 minutes (at 20°C)

- Good flow characteristics

- Shrinkage compensated (both in plastic and hardening stage)

- No segregation or bleeding

- IC price 0.21 Euro/kg

Please inform us of your interest as soon as possible and the quantity of material required.

SETTING A FIELD TEST PLAN

- Assign a responsible Product Engineer

- Local

- Know the information required from a field test (refer to template)

- Workability, mixing, open time, visual aspects, flow characteristics

- Application techniques (manual  & mechanical)

- Checking the applications limits

- Comparison to competitor or Sika product

- Customer benefits

- Identify cost and margin assumptions for the product

- Please provide average sales price of competitor benchmark if available.

Test Certificates

- By Sika Austria – ITT report for CE marking is in preparation

For requesting SikaGrout®-334 from Austria:

Material request please contact Mr. Werner Peccoraro 
email : peccoraro.werner@at.sika.com 
Tel.   +43 506 10 1204
Fax. +43 506 10 1906

Or Mr. Christian Sauerwein
email : sauerwein.christian@at.sika.com
Tel.   +43 506 10 1204
Fax. +43 506 10 1906

For technical inquiries contact Mr. Philipp Maehr
email : maehr.philipp@at.sika.com
Tel.   +43 506 10 1310
mobile: + 43 5552610123

For further information or clarifications, do not hesitate to contact me

Mähr Philipp